C.A.T.A.S.T.R.O.P.H.E.™ Tablet Production RFQ

Quote-ready specification sheet for rugged emergency-response tablets

| Issuing Organization | Dingfelder Enterprises Walter W. Dingfelder North East, PA 16428 dingfelder.co walter@dingfelder.co (814) 440-5753 |
| --- | --- |

# 1. RFQ Objective

Dingfelder Enterprises is requesting production quotes for rugged C.A.T.A.S.T.R.O.P.H.E. tablets used as emergency-response endpoints in A.I.R.O.N. deployments. Vendors shall quote both Android and Windows options where available. Each option must meet the mandatory communications, location, kiosk, security, power, and ruggedness requirements listed below.

The tablet is not a general-purpose consumer device. It is a locked, emergency-ready industrial device intended to remain available during facility incidents, communications failures, machine events, safety events, responder handoffs, and tablet-finder recovery scenarios.

# 2. Quote Options Requested

| Option | Build Type | Required Quote Scope |
| --- | --- | --- |
| A | Android rugged kiosk tablet | Primary recommended path. Quote rugged Android Enterprise/dedicated-device capable tablets with full mandatory communications hardware, dock, charger, provisioning, and fleet management options. |
| B | Windows rugged kiosk tablet | Customer-IT alternate path. Quote rugged Windows 11 Pro/Enterprise/IoT-capable tablets with assigned-access/kiosk support, full mandatory communications hardware, dock, charger, provisioning, and fleet management options. |
| C | Enhanced emergency field unit | Optional upgrade concept. Quote high-visibility shell, handle, strap, emergency label set, asset serialization, enhanced speaker/alert hardware, and wall-dock package if available. |

# 3. Mandatory Communications and Location Baseline

The following capabilities are mandatory on every production tablet. They shall not be quoted as optional add-ons unless the vendor is separating base tablet cost from accessory or service-plan cost. Any exception must be clearly identified as a non-compliance item.

| Capability | Status | Required Purpose | Vendor Must Identify |
| --- | --- | --- | --- |
| Wi-Fi | Mandatory | Starlink connection, VOIP, A.I.R.O.N./V.A.U.L.T. sync, software updates during approved maintenance windows. | Wi-Fi version, supported bands, security support, roaming behavior. |
| Bluetooth | Mandatory | Alternate short-range connection path to the A.I.R.O.N. base when Wi-Fi is unavailable. | Bluetooth version, BLE/classic support, pairing controls, supported profiles/APIs. |
| GPS/GNSS | Mandatory | Constant tablet location, tablet finder, responder awareness, Google Maps/facility map projection, last-known recovery. | Supported constellations, location accuracy, indoor/assisted modes, update interval support. |
| LTE/5G Cellular | Mandatory | Off-site emergency data fallback, SMS path, tablet tracking, facility/responder communications when Wi-Fi/Starlink/local infrastructure is unavailable. | LTE minimum, 5G preferred; carrier bands, SIM/eSIM support, carrier certification, antenna options. |
| SMS/Text Messaging | Mandatory | Emergency text alerts, responder/user notification, fallback communication, tablet-finder messages. | Native cellular SMS, carrier SMS, or approved gateway; queue/resend behavior; logging support. |
| Frequent Location Snapshot | Mandatory | Persistent last-known tablet location with timestamp, battery, connection state, and device status. | Configurable refresh intervals, offline storage method, export/API capability. |

# 4. Emergency Communications Behavior

Wi-Fi shall support Starlink-based internet connectivity and VOIP calling or push-to-talk workflows.

Cellular shall provide independent off-site fallback for emergency data, SMS, location reporting, and fleet visibility.

SMS shall function as an emergency communication channel when Wi-Fi/VOIP is not available.

Bluetooth shall provide a local fallback path between the tablet and the A.I.R.O.N. base station if Wi-Fi is unavailable.

The tablet shall maintain and refresh a last-known-location snapshot frequently. The snapshot shall include tablet ID, timestamp, GPS/GNSS position, battery state, communication state, and device status.

Failed outbound messages or event updates should queue locally and transmit automatically when a valid communication path returns.

Vendor shall identify whether the tablet supports cellular voice, cellular data only, SMS, eSIM, physical SIM, dual SIM, or carrier-specific activation requirements.

# 5. Hardware Requirements - Shared Baseline

| Category | Minimum Requirement | Preferred Requirement | Vendor Response Needed |
| --- | --- | --- | --- |
| Display | 10-inch class touch display | 10-12 inch, 700-1000 nits, outdoor readable | Size, brightness, glass type, glove/wet-touch support. |
| Ruggedness | Industrial rugged tablet, IP65 class, 4 ft drop class | IP67, MIL-STD-810H class, 5-6 ft drop class | Ingress rating, drop rating, certifications, test standard. |
| Battery | 6-hour usable runtime | Full-shift battery, hot-swap or extended battery option | Battery Wh, runtime, charging time, hot-swap details. |
| Charging | USB-C or dedicated charger | Wall dock, desktop dock, multi-bay dock, vehicle dock option | Dock options and per-unit cost. |
| Camera | Rear camera 8 MP | 13 MP+ rear camera, autofocus, barcode/QR capable | Camera specs, SDK availability, scanner option. |
| Audio | Speaker and microphone | Loud industrial speaker, noise handling, VOIP-ready mic | Speaker dB if available, mic/noise details. |
| Alerts | Visual/audible notifications | Haptic, bright visual indicators, optional external beacon | Alert capabilities and accessory options. |
| Physical handling | Hand strap or case available | Handle, shoulder strap, glove-friendly grip, high-visibility casing | Accessories and pricing. |
| Environmental | Normal industrial indoor operation | Wide temperature operation and humidity resistance | Operating temp range, storage temp, humidity rating. |
| Serviceability | Replaceable charger/accessories | Replaceable battery, field-swap accessories, documented repair path | Warranty, repair process, spare parts availability. |

# 6. Android Option - Required Response

Quote rugged Android tablets capable of enterprise/dedicated-device kiosk operation. Android is the preferred production direction when a locked, single-purpose emergency endpoint is the primary goal.

| Item | Requirement / Response Needed |
| --- | --- |
| Operating system | Android Enterprise capable. Identify Android version, lifecycle/support window, and security update policy. |
| Kiosk mode | Dedicated-device or equivalent kiosk mode required. Vendor shall identify supported MDM/provisioning method. |
| Processor/RAM/storage | Minimum 6 GB RAM / 128 GB storage. Preferred 8 GB+ RAM / 256 GB storage. |
| Update control | No surprise production updates. Updates shall be controllable by policy and scheduled for maintenance windows. |
| Provisioning | Quote repeatable provisioning/re-provisioning process for fleet setup, replacement devices, and pilot rollout. |
| Security | Encryption, restricted settings, restricted app installation, restricted notifications, remote lock/wipe if managed. |

# 7. Windows Option - Required Response

Quote rugged Windows tablets for customers who prefer Windows, Microsoft management, Windows-based support practices, or customer IT alignment.

| Item | Requirement / Response Needed |
| --- | --- |
| Operating system | Windows 11 Pro, Windows 11 Enterprise, or Windows IoT Enterprise. Identify version and support lifecycle. |
| Kiosk mode | Assigned-access/single-app or restricted shell support required. Identify exact implementation. |
| Processor/RAM/storage | Minimum Intel N100/N200 class or equivalent, 8 GB RAM, 128 GB SSD. Preferred i5-class, 16 GB RAM, 256 GB SSD. |
| Update control | No surprise production updates. Windows updates shall be controllable and scheduled for maintenance windows. |
| Provisioning | Quote imaging/provisioning process, recovery media, replacement tablet setup, and fleet management compatibility. |
| Security | BitLocker/device encryption where supported, local admin lockdown, app restrictions, browser/settings restrictions. |

# 8. Kiosk, Security, and Fleet Management Requirements

Tablet must boot or wake directly into the approved C.A.T.A.S.T.R.O.P.H.E. app experience without exposing general consumer functions.

Settings, app store, web browsing outside the allowed environment, unrelated notifications, and user-driven configuration changes must be blocked or controlled.

Device must support encryption, authentication policy, asset identity, serial number tracking, remote lock/wipe where supported, and logged configuration changes.

Vendor shall identify MDM compatibility, included management software, license cost, per-device management cost, and any recurring fees.

Vendor shall identify how offline operation is handled when Wi-Fi, cellular, or Starlink is unavailable.

# 9. Location, Tablet Finder, and Mapping Requirements

GPS/GNSS shall support persistent tablet location and tablet-finder workflows.

Location snapshots shall be refreshed frequently during active/emergency use and at a configurable interval during standby/fleet monitoring.

Each snapshot shall include tablet ID, timestamp, location, accuracy if available, battery state, Wi-Fi/cellular/Bluetooth state, and last communication path.

Tablet location shall be usable for Google Maps projection and future facility-map/digital-twin projection.

Vendor shall identify any limitations to GPS/GNSS indoors, assisted-GPS behavior, and cellular/Wi-Fi location fallback behavior.

# 10. Accessories and Optional Adders to Quote

| Accessory / Adder | Required or Optional | Quote Notes |
| --- | --- | --- |
| Wall-mounted charging dock | Required quote line | Quote per tablet and multi-unit facility pricing. |
| Desktop charging dock | Required quote line | Quote if separate from wall dock. |
| Multi-bay charger | Required quote line | Quote for pilot and fleet staging. |
| Spare battery set | Required quote line if removable battery | Quote 1 spare battery per tablet and 2 spares per tablet. |
| Hand strap / shoulder strap / handle | Required quote line | Emergency field use and industrial handling. |
| Rugged case / high-visibility shell | Required quote line | Prefer bright emergency color option if available. |
| Screen protector | Required quote line | Quote installed and replacement packs. |
| Vehicle mount | Optional adder | Responder/facility vehicle option. |
| Barcode/QR scanner | Optional adder | SDS/MSDS, asset ID, incident tagging. |
| External alert/speaker/beacon | Optional adder | Emergency alert visibility/noise environments. |
| Branding and serialization | Required quote line | Boot screen, label, asset tag, QR asset ID, case marking. |

# 11. Carrier and SMS Contract Questions

Dingfelder Enterprises intends to pursue mass/enterprise cellular contract options with nationwide carriers. Hardware quotes shall clearly identify carrier compatibility and activation requirements.

Which U.S. carrier bands are supported?

Is the device carrier-certified? If yes, identify carrier certifications.

Does the device support physical SIM, eSIM, or both?

Does the device support native SMS? If not, identify the approved SMS gateway path.

Can SMS include tablet ID, GPS/GNSS location, timestamp, battery state, and emergency event code?

Can failed SMS/data messages queue and resend automatically?

Does the vendor offer or support enterprise carrier procurement, activation, staging, or lifecycle management?

What recurring monthly costs should be expected for device management, cellular service support, or SMS gateway use?

# 12. Production Quantities and Price Breaks

| Quantity | Purpose | Vendor Price Requested |
| --- | --- | --- |
| 1 unit | Prototype/sample evaluation | Unit price, lead time, freight, setup charges. |
| 5 units | Pilot facility package | Unit price, provisioning, docks, accessories, management costs. |
| 10 units | Small deployment | Price break and production lead time. |
| 25 units | First production run | Price break and production lead time. |
| 50 units | Scaled production | Price break and production lead time. |
| 100 units | Volume target | Price break, capacity, support model. |

# 13. Required Quote Line Items

Base Android rugged tablet meeting the mandatory communications baseline.

Base Windows rugged tablet meeting the mandatory communications baseline.

Docking/charging package per tablet and per facility.

Spare battery package, if removable batteries are supported.

Rugged handling package: case, strap, handle, screen protector, high-visibility option.

MDM/provisioning/kiosk setup cost, including per-device recurring costs if applicable.

Branding, serialization, QR asset tag, boot/splash screen, and labeling cost.

SMS/cellular enablement requirements and any recurring platform/gateway costs.

Warranty, extended warranty, replacement device process, repair turnaround, and spare parts package.

Prototype lead time, pilot lead time, production lead time, and volume capacity.

# 14. Acceptance Test Requirements for Sample Unit

| Test | Pass Requirement | Notes / Evidence Requested |
| --- | --- | --- |
| Cold boot | Device boots directly into locked C.A.T.A.S.T.R.O.P.H.E. kiosk experience. | Video or live demonstration preferred. |
| Wake from sleep | Device wakes without exposing settings, app store, browser, or unrelated apps. | No login confusion during emergency use. |
| Wi-Fi/Starlink path | Device connects over Wi-Fi and supports VOIP/data workflows. | Demonstrate connection and audio path. |
| Cellular fallback | Device sends/receives emergency data path over cellular with Wi-Fi disabled. | Identify carrier/SIM used. |
| SMS | Device sends emergency SMS including tablet ID, timestamp, battery, and location. | Show native or gateway behavior. |
| GPS/GNSS | Device reports location and refreshes last-known snapshot. | Show map projection or exported snapshot. |
| Bluetooth fallback | Device connects to a local A.I.R.O.N. base or test endpoint with Wi-Fi disabled. | Show supported Bluetooth method/API. |
| Offline operation | Device retains local emergency content and queues failed updates/messages. | Demonstrate after disabling Wi-Fi/cellular. |
| Battery | Device meets quoted runtime and charges reliably in dock. | Provide battery report and dock cycle. |
| Rugged handling | Device meets quoted rugged/drop/IP rating. | Provide certification/test documentation. |
| Re-provisioning | Replacement tablet can be restored from documented provisioning process. | Required for fleet support. |

# 15. Vendor Response Format

Please return the following information with the quote:

Manufacturer and model number for each quoted Android and Windows tablet.

Detailed specification sheet for each model, including wireless, cellular, GPS/GNSS, ruggedness, display, battery, camera, and docking details.

Completed compliance notes against each mandatory requirement above.

Itemized pricing for prototype, 5, 10, 25, 50, and 100 unit quantities.

Separate pricing for accessories, docks, chargers, spare batteries, MDM/provisioning, branding, and support.

Lead times for sample, pilot, and production quantities.

Warranty, repair, replacement, and support terms.

Carrier compatibility and cellular/SMS provisioning notes.

Any exceptions, non-compliant items, required substitutions, or recommended upgrades.

# 16. Cover Email Template

Subject: RFQ - Rugged C.A.T.A.S.T.R.O.P.H.E. Emergency Tablets - Android and Windows Options

Hello,

Dingfelder Enterprises is requesting production pricing for rugged emergency-response tablets for our C.A.T.A.S.T.R.O.P.H.E. / A.I.R.O.N. deployment hardware.

We are requesting both Android and Windows rugged tablet options. Please treat Wi-Fi, Bluetooth, GPS/GNSS, LTE/5G cellular, and SMS/text messaging as mandatory baseline hardware/capability requirements, not optional add-ons.

Please quote prototype, 5, 10, 25, 50, and 100 unit quantities, including docks, charging accessories, straps/cases, spare batteries where applicable, kiosk/provisioning support, MDM/fleet-management options, branding/serialization, warranty, lead time, and any recurring software or cellular/SMS costs.

Please also identify carrier compatibility, SIM/eSIM support, SMS implementation, and whether you can support enterprise or nationwide carrier fleet activation.

Thank you,

Walter W. Dingfelder

Dingfelder Enterprises

walter@dingfelder.co | (814) 440-5753

# 17. Immediate RFQ Decision Notes

Quote Android as the preferred production standard for a locked, purpose-built emergency tablet.

Quote Windows as the customer-IT alternate standard for facilities that require Microsoft/Windows alignment.

Do not accept any quote that omits cellular, SMS, Wi-Fi, Bluetooth, or GPS/GNSS from the baseline tablet package without a clearly documented exception.

Treat wall-docking, charging, handling accessories, provisioning, and asset serialization as part of the production package, not afterthoughts.